[ Internet of things ]

Practical-10

-:AIM:-

Raspberry pi basic Programming with Ultrasonic Sensor.

Submitted By:  Dharmay Sureja

Enrollment No:17012011056

GANPAT UNIVERSITY

U. V. Patel College of Engineering

Computer Engineering Department

AIM:- Raspbian pi basic programming with Ultrasonic Sensors.

Experiments

- Measure the distance of an obstacle using ultrasonic sensor.

Components used :  Breadboard  , 2.2K Ohm Resistor, 1K Ohm Resistor , Raspberry pi ,

SD Card & Adapter , Jumper Wire , Ultrasonic Sensor , USB Cable

Circuit:

Code : distance.py

import RPi.GPIO as GPIO

import time

GPIO.setmode(GPIO.BCM)

GPIO_TRIGGER = 23

GPIO_ECHO = 24

GPIO.setup(GPIO_TRIGGER, GPIO.OUT)

GPIO.setup(GPIO_ECHO, GPIO.IN)

def distance():

GPIO.output(GPIO_TRIGGER, True)

# set Trigger after 0.01ms to LOW

time.sleep(0.00001)

GPIO.output(GPIO_TRIGGER, False)

StartTime = time.time()

StopTime = time.time()

while GPIO.input(GPIO_ECHO) == 0:

StartTime = time.time()

# save time of arrival

while GPIO.input(GPIO_ECHO) == 1:

StopTime = time.time()

# time difference between start and arrival

TimeElapsed = StopTime - StartTime

# multiply with the sonic speed (34300 cm/s)

# and divide by 2, because there and back

distance = (TimeElapsed * 34300) / 2

return distance

if __name__ == '__main__':

try:

while True:

dist = distance()

print ("Measured Distance = %.1f cm" % dist)

time.sleep(1)

# Reset by pressing CTRL + C

except KeyboardInterrupt:

print("Measurement stopped by User")

GPIO.cleanup()

- Send SMS if Object distance is less then 10 cm ( using Twillo API )

Code : send_sms.py

import RPi.GPIO as GPIO

import time

from twillo.rest import Client

account_sid = "AC92b0aeee716d654f31561ac68bec0f12"

auth_token = "95aa6dee3decb6a6c9fb224e4d7c4916"

GPIO.setmode(GPIO.BCM)

GPIO_TRIGGER = 23

GPIO_ECHO = 24

GPIO.setup(GPIO_TRIGGER, GPIO.OUT)

GPIO.setup(GPIO_ECHO, GPIO.IN)

def distance():

GPIO.output(GPIO_TRIGGER, True)

# set Trigger after 0.01ms to LOW

time.sleep(0.00001)

GPIO.output(GPIO_TRIGGER, False)

StartTime = time.time()

StopTime = time.time()

while GPIO.input(GPIO_ECHO) == 0:

StartTime = time.time()

# save time of arrival

while GPIO.input(GPIO_ECHO) == 1:

StopTime = time.time()

# time difference between start and arrival

TimeElapsed = StopTime - StartTime

# multiply with the sonic speed (34300 cm/s)

# and divide by 2, because there and back

distance = (TimeElapsed * 34300) / 2

return distance

if __name__ == '__main__':

try:

while True:

dist = distance()

print ("Measured Distance = %.1f cm" % dist)

time.sleep(1)

if dist < 10:

client =  Client(account_sid,auth_token)

message = client.message.create(

to = "+917845612390"

from = "575701"

body = "Hello from GNU’s Rasp_PI")

print(message.sid)

quit(0)

# Reset by pressing CTRL + C

except KeyboardInterrupt:

print("Measurement stopped by User")

GPIO.cleanup()

- Open www.twillo.com

- Register with your mail and mobile Number .

- Login www.twillo.com with your email address

- Note the Authentication ID and Authentication Token.

- Get twillo phone number (In order to make calls or send Messages through thw twillo

API , you need to get a Twillo Phone Number .